# enchid

This is the readme file for the enchid firmware for general-purpose HID-class USB communications. The firmware is for Cypress Semiconductors' CY7C637xx Encore series of USB peripherals.

Enchid is brought to you by

Jan Axelson

http://www.lvr.com

jan@lvr.com

## Files

Enchid.zip includes the following files:

### The Readme File

readme.doc is this file.

### The PC’s Inf File

Usbhidio.inf is an INF file that tells Windows 98 what drivers to use for the device. The device uses Windows’ built-in HID drivers. You can view the file in any text editor. Usbhidio.inf is optional. If you don't use it, Windows 98 will use the system file hiddev.inf for the device. Windows 2000 and Windows XP use the system file input.inf.

### The Device Code

In the directory Encore:

Enchid.asm is assembly-code firmware for Cypress’ CY7C63743 USB microcontroller. The code handles enumeration on attachment to a host, and can exchange Input reports using control or interrupt transfers, Output reports using control transfers, and IN and OUT Feature reports using control transfers. The firmware conforms to the USB’s human-interface device (HID) class specification.

USB.inc is assembly code containing USB descriptor and request constants

637xx.inc is assembly code containing register assignments and other chip-specific equates.

Enchid.rom is the assembled code for loading into the development board’s memory.

Enchid.lst is the listing file produced by the assembler.

Enchid.hex is the Hex file produced by the assembler. You can use the file with device programmers that accept Intel Hex files or with the development system.

## What You Need

To use this code, you need all of the following:

- A PC running Windows 98 or Windows 2000.

- Cypress’ CY3654 development kit, which contains a development board, assembler, and the cydb monitor program for loading and running the firmware. For more information about the kit and Cypress chips, see www.cypress.com.

- A host application to communicate with the firmware. From www.lvr.com, you can download the following projects for use with enchid:

hidvb.zip contains a Visual-Basic project.

usbhidioc.zip contains a Visual C++ project.

- An understanding of USB and HID communications. See http://www.lvr.com for links to more information about USB and HIDs.

## Also helpful are:

- The freeware USBCheck and HIDView utilities, available from http://www.usb.org . These will tell you if the host successfully enumerated the device, and if so, will display the descriptors retrieved.

- Oscilloscope, logic analyzer, voltmeter, protocol analyzer. The USB transmissions are encoded in a way that makes it just about impossible to interpret the data without a specialized protocol analyzer, but basic test equipment is still useful for debugging. For example, the firmware includes debug code that sets port bits when the code is in a particular routine.

## How to Use the Code

1. In Windows 98 only, copy usbhidio.inf to your \Windows\Inf directory (optional).

2. Use the Cypress cydb program to load enchid.rom or enchid.hex into the development board’s memory. Click the monitor program's Run button. If the Device Manager displays a window that says that new hardware has been detected, let it search for a driver. In Windows 98, specify searching the \Windows\Inf folder.

3.In Windows 98, look for the device under Start Menu > Settings > Control Panel > System > Device Manager > Human Interface Devices. In Windows 2000, look under Start Menu > Settings > Control Panel > System > Hardware > Device Manager. The device should be listed as  both “HID-compliant device” and “Sample USB Human Interface Device (HID)” (or "USB Human Interface Device" if you don't use usbhido.inf).

4. Run your host application.

5. From this point, you can edit and experiment with the host and Cypress code as you wish. Enchid.asm contains the source code for the firmware. Edit it with any text editor, save as plain text, re-assemble using Cypress’ Cyasm assembler, and use the monitor program to load the new version into the development board.

6. Send any comments, questions, bug reports, etc. to me at jan@lvr.com.

## Tips

- When you run USBCheck and HIDView, Windows detects your device as a “USB Diagnostic Device” (under Start Menu > Control Panel > System > Device Manager > Other Detected Devices. When you’re finished using the utilities, unplug and replug the USB cable to allow Windows to re-enumerate the board as a standard Human Interface Device.

### Tips for using Cyasm (the Cypress assembler):

- Variable names are case-sensitive. Port0_data is not the same as Port0_Data.

- The “~” signifies complement. For example, The complement of the variable “test” is “~test”. This is undocumented.

### Tips for using the CY3654 Development Kit:

- Be sure to plug the USB cable into the target board (the Applications board that connects to the Personality module). The USB connector on the main development board is unused.

## Rights information

The computer software provided in the enchid.zip file is copyright 1999, 2000, and 2004 by Jan Axelson. The software is provided as-is. You have a royalty-free right to use, modify, reproduce and distribute this software (and/or any modified version) in any way you find useful, provided that you agree that Jan Axelson and Lakeview Research have no warranty, obligation or liability for their contents!

I welcome any comments, bug reports, questions, etc. about this code. Contact me at jan@lvr.com .

Jan Axelson