# Usbhidio

This is the readme file for the usbhidio application, which includes host software and device firmware for communicating with USB HID-class devices.

Usbhidio is brought to you by

Jan Axelson

Lakeview Research

2209 Winnebago St.

Madison, WI 53704

Phone: 608-241-5824

Fax: 608-241-5848

http://www.lvr.com

jan@lvr.com

(Usbhidio means Universal Serial Bus (usb) Human Interface Device (hid) Input/Ouput (io) application.)

## Files

Usbhidio includes the following files:

### The Readme File

readme.doc is this file.

### The PC’s Inf File

Usbhidio.inf is an INF file that tells Windows what drivers to use for the device. The device uses Windows’ built-in HID drivers. You can view the file in any text editor.

### The Visual-Basic Project

In the directory VisualBasic:

Usbhidio.vbp is a Visual-Basic 6 project that finds the device and exchanges data with it.

Apideclarations.bas contains declarations for the Win32 API functions used in the project.

frmMain.frm is the project’s form.

usbhidio.exe is the executable file created from usbhidio.vbp.

### The Device Code

In the directory CY7C63000:

Usbhidio.asm is assembly-code firmware for Cypress’ CY763001 USB microcontroller. The code handles enumeration on attachment to a host, and can exchange data in both directions. The firmware conforms to the USB’s human-interface device (HID) class specification.

Usbhidio.rom is the assembled code for loading into the development board’s memory.

Usbhidio.lst is the listing file produced by the assembler.

Usbhidio.hex is the Hex file produced by the assembler (for use with device programmers that accept Intel Hex files).

## What You Need

To use this code, you need all of the following:

- A PC running Windows 98.

- Visual Basic 6, to load usbhidio.vbp.

- Cypress’ 3650 development kit, which contains a development board, assembler, and a monitor program for loading and running the usbio firmware, or the Starter Kit (see Tips below).

- An understanding of USB and HID communications. See http://www.lvr.com for links to more information about USB and HIDs.

### Also helpful are:

- The freeware USBCheck and HIDView utilities, available from http://www.usb.org . These will tell you if the host successfully enumerated the device, and if so, will display the descriptors retrieved. HIDView can also read data from the device.

- Oscilloscope, logic analyzer, voltmeter, protocol analyzer. The USB transmissions are encoded in a way that makes it just about impossible to interpret the data without a specialized protocol analyzer, but basic test equipment is still useful for debugging. For example, the firmware includes debug code that sets port bits when the code is in a particular routine.

- The HID documentation in the Windows 98 DDK, in the WDM section. You can download the DDK from Microsoft.

## How to Use the Code

1. Copy usbhidio.inf to your \Windows\Inf directory.

2. Set the jumpers in the CY3650 board like this (thanks to John Musselman for recommending including this):

SW1-1    Open        (Enable Bus Reset)

SW1-2 Closed

SW1-3 Closed

SW1-4 Closed

SW1-5 Closed

SW1-6 Open

SW1-7 Open

SW1-8 Open

SW3-1 Open        (Program runs in RAM)

SW3-2 Open        (Run on Reset)

SW3-3 Closed

SW3-4 Closed

SW3-5 Closed

SW3-6 Closed

SW3-7 Closed

SW3-8 Closed

3. Use the Cypress monitor program to load usbhidio.rom into the development board’s memory. Press the Start button on the development board.

4. Close the monitor program.

5. Run USBCheck. Unplug and plug in the cable. Run the tests. View the results.

6. Run the HIDView tests in USBCheck. View the results.

7. Unplug and replug the cable. Look for the device under Windows 98’s Start Menu > Control Panel > System > Device Manager > Human Interface Devices. The device should be listed as    both “HID-compliant device” and “Sample USB Human Interface Device (HID)”.

8. Run Visual Basic. Load the usbhidio project. Run and view the results.

9. From this point, you can edit and experiment with the Visual-Basic and Cypress code as you wish. Usbhidio.asm contains the source code for the firmware. Edit it with any text editor, save as plain text, re-assemble using Cypress’ Cyasm assembler, and use the monitor program to load the new version into the development board.

10. Send any comments, questions, bug reports, etc. to me at jan@lvr.com.

## Tips

- When you run USBCheck and HIDView, Windows detects your device as a “USB Diagnostic Device” (under Start Menu > Control Panel > System > Device Manager > Other Detected Devices. When you’re finished using the utilities, unplug and replug the USB cable to allow Windows to re-enumerate the board as a standard Human Interface Device.

- ReadFile is a blocking call. If the device has nothing to send, the Visual-Basic application will hang while it waits. The firmware is written so the device always has something to send. But if the application does hang, you can often restore control by unplugging and replugging the USB cable. If that fails, press control-alt-delete and wait for the dialog box that enables you to close the application.

- The firmware will run on the Starter Kit. Use the EPROM programmer to load usbhidio.hex into a chip, insert the chip into the kit's socket, and attach the board to the bus. The code currently will fit in a 2k CY7C63000, or a 4k '63001.

- To prevent errors, close the Cypress monitor program before you run USBCheck or HIDView.

- If your PC doesn’t boot with the development board attached, unplug the USB cable, boot up, and reload the firmware.

- After loading new firmware on the development board, you must press the board's Start button, then unplug and replug the USB cable so that Windows will detect and enumerate the device with the new firmware.

### Tips for using Cyasm (the Cypress assembler):

- Variable names are case-sensitive. Port0_data is not the same as Port0_Data.

- The “~” signifies complement. For example, The complement of the variable “test” is “~test”. This is undocumented.

## Rights information

The computer software provided in the usbhidio.zip file is copyright 1999 by Jan Axelson. The software is provided as-is. You have a royalty-free right to use, modify, reproduce and distribute this software (and/or any modified version) in any way you find useful, provided that you agree that Jan Axelson and Lakeview Research have no warranty, obligation or liability for their contents!

I welcome any comments, bug reports, questions, etc. about this code. Contact me at jan@lvr.com .

Jan Axelson

Lakeview Research